FPGA-Based HLS Acceleration for the Ambient Intelligence Radar Pipeline

Feasibility Study and Implementation Report

Omar Elkhateeb  •  Ambient Intelligence  •  University of Minnesota  •  March 2026

Target: Artix-7 xc7a35tcpg236-1 (Basys-3)  |  Tool: Vitis HLS 2025.1  |  Clock: 15 ns (67 MHz)

# 1. Executive Summary

This report presents results from an HLS feasibility study for the Ambient Intelligence fall detection radar system (TI IWR6843AOP, 60 GHz mmWave). Two FPGA kernels were implemented, verified against the Python golden reference, and synthesized with AXI interfaces for Vivado integration:

Kernel 1 — tlv_sph2cart_axi: Fused pipeline that decompresses compressed radar point cloud data and converts spherical→Cartesian in a single streaming pass. AXI-Stream data, AXI-Lite control. II=1.

Kernel 2 — fall_detect_axi: Stateful fall detection with BRAM-backed circular buffers, multi-criteria evaluation, cooldown logic. AXI-Lite interfaces. II=1.

Both kernels fit on the Basys-3 simultaneously (18% DSP, 16% BRAM, 15% FF, 93% LUT).

# 2. System Overview

The IWR6843AOP radar performs on-chip signal processing and transmits tracked output via UART at 921,600 baud to a Raspberry Pi. The sensor runs at ~18 Hz (55 ms frame period) with 70° FOV and up to 30 tracked targets. The Python pipeline consists of: datastream.py (UART I/O and magic word sync), parseFrame.py (frame header parsing and TLV dispatch), parseTLVs.py (25+ TLV parsers including compressed point cloud decompression), gui_common.py (spherical-to-Cartesian coordinate conversion), new_fall_detection.py (multi-criteria fall detection with height history), and visualization modules (people_tracking.py, Visualizer.py). Supporting files include tlv_defines.py (TLV type constants), configuration files (.cfg), and main.py (application entry point).

# 3. HLS Candidate Analysis

| Module | Suitability | Rationale |
| --- | --- | --- |
| gui_common.py (sphericalToCartesian) | Excellent | Pure trig math, no dependencies between points, maps to CORDIC |
| parseTLVs.py (compressedSpherical) | Excellent | Sign extension + decompression + trig fusible into one pipeline |
| new_fall_detection.py (step) | Moderate | Stateful logic; O(n²) TID match → O(1) BRAM lookup in hardware |
| parseFrame.py | Not suitable | Control-flow dispatcher, no compute to accelerate |
| datastream.py | Not suitable | Serial I/O, hardware-bound by 921,600 baud UART |
| parseTLVs.py (other parsers) | Not suitable | Pure struct unpacking — memory copy, not compute |
| people_tracking.py, Visualizer.py | Not suitable | GUI/visualization code, inherently CPU/GPU |

# 4–6. Point Cloud Kernel: Optimization Progression

The spherical-to-Cartesian kernel was developed in three phases. Phase 1 used float types to validate correctness (max error 9.54e-07 vs Python). Phase 2 migrated to ap_fixed types sized for DSP48E1 (18-bit), replacing Xilinx float IP with CORDIC — cutting DSP from 67 to 9 (–86%). Phase 3 fused the TLV decompression from parseTLVs.py into the same loop, eliminating the intermediate spherical array. The compressed point format is 64 bits: int8 elevation, int8 azimuth, int16 doppler, uint16 range, uint16 SNR. In hardware, ap_int provides automatic sign extension, replacing Python’s manual overflow checks.

| Metric | Phase 1 (Float) | Phase 2 (ap_fixed) | Phase 3 (Fused) |
| --- | --- | --- | --- |
| II | 1 | 1 | 1 |
| Iter. Latency | 29 cycles | 21 cycles | 23 cycles |
| DSP48E1 | 67 (74%) | 9 (10%) | 14 (16%) |
| FF | 10,165 (24%) | 3,292 (8%) | 3,897 (9%) |
| LUT | 12,002 (58%) | 11,041 (53%) | 11,231 (54%) |
| Max Error | 9.54e-07 | 1.43e-03 | 1.11e-03 |
| Functionality | sph2cart only | sph2cart only | Decompress + sph2cart |

# 7. Fall Detection Kernel

The fall detection kernel ports new_fall_detection.py to hardware. Key HLS translations: Python’s nested O(n²) TID matching becomes O(1) BRAM address lookup; deque(maxlen=82) becomes a circular buffer with write pointer; time.time() becomes a frame counter (182 frames = 10 sec cooldown); the speed buffer is eliminated (only 2 most recent heights needed); and set difference for track reset becomes a single-cycle bitmask operation.

Initial synthesis reported II=2 due to 3 BRAM accesses per cycle (read oldest, read previous, write new). Fix: store previous height in a 30-element register array, reducing BRAM accesses to 2 (dual-port compatible). Verified across 5 scenarios: standing (no false alerts), falling (correct detection at frame 8), cooldown (no re-trigger), track disappearance/reappearance (clean reset), and two simultaneous tracks (independent detection). All passed.

| Metric | Value | Basys-3 Limit |
| --- | --- | --- |
| PROCESS_HEIGHTS II | 1 | — |
| DSP48E1 | 2 | 90 (2%) |
| BRAM18K | 4 | 50 (8%) |
| FF | 2,577 | 41,600 (6%) |
| LUT | 7,546 | 20,800 (36%) |

# 8. AXI Interface Integration

tlv_sph2cart_axi uses AXI-Stream for data (64-bit compressed in, 128-bit packed [X,Y,Z,Doppler,SNR] out with TLAST) and AXI-Lite for configuration (unit factors, point count, start/done). fall_detect_axi uses all AXI-Lite (processor writes heights, starts kernel, reads 30-element result array). Both synthesized at 15 ns (67 MHz):

| Resource | tlv_sph2cart_axi | fall_detect_axi | Combined | Basys-3 |
| --- | --- | --- | --- | --- |
| DSP48E1 | 14 | 2 | 16 | 90 (18%) |
| BRAM18K | 0 | 8 | 8 | 50 (16%) |
| FF | 3,275 | 2,833 | 6,108 | 41,600 (15%) |
| LUT | 11,595 | 7,801 | 19,396 | 20,800 (93%) |

# 9–10. Deliverables and Files

All source code is on the HLS branch of the GitHub repository (github.com/ambientintel/Ambient-Intelligence/tree/HLS), organized as: hls_kernels/ (Phase 1 float), hls_kernels_v2/ (Phase 2 ap_fixed), hls_kernels_v3/ (Phase 3 fused), hls_fall_detect/ (fall detection), and hls_axi/ (final AXI versions). Each directory contains the .h/.cpp kernel, testbench, test vector generator (where applicable), and Vitis HLS config file.

# 11. Remaining Work

Vivado Integration: Export HLS IPs, create block design with MicroBlaze or Zynq PS, connect AXI buses. UART-to-AXI Bridge: Custom IP to parse UART frames and feed compressed TLV data into the AXI-Stream port. Live Testing: Run real sensor data through the FPGA pipeline in the BMDC and compare against the Python pipeline. Performance Benchmarking: Measure end-to-end latency (FPGA kernels complete in ~1.5 µs vs. the 55 ms frame period — a 36,000× margin). Platform Migration: Retarget to Zynq-7000 or larger Artix-7 to resolve tight LUT fit on the Basys-3.

# 12. Conclusion

Both the radar data path (compressed point cloud decompression + spherical-to-Cartesian conversion) and fall detection algorithm were successfully accelerated to II=1 using Vitis HLS. The fixed-point migration reduced DSP from 74% to 16% while maintaining sub-millimeter accuracy. Both AXI-interfaced kernels fit on the Basys-3 simultaneously and are ready for Vivado block design integration. The remaining work is engineering integration (Vivado, UART bridge, live testing) rather than research.